## Logic Explanation

The automated object sorting system operates using a sequential, event-driven control logic. The system continuously monitors sensor inputs, makes deterministic decisions based on those inputs, and actuates mechanical components accordingly. The logic is designed to be simple, reliable, and suitable for real-time robotic applications.

### 1. System Initialization Logic

At startup, the microcontroller performs the following actions:

- Configures all sensor and actuator pins.

- Sets the TCS3200 color sensor to a fixed frequency scaling for stable readings.

- Attaches all servo motors to their respective control pins.

- Initializes servo positions to a neutral (home) state.

- Starts the conveyor motor to allow object flow.

This ensures the system begins operation in a known and safe state.

### 2. Object Detection Logic

The system continuously checks the IR sensor:

- IR sensor LOW → Object is present on the conveyor.

- IR sensor HIGH → No object detected.

When an object is detected:

- The conveyor motor is stopped immediately.

- A short delay is applied to eliminate motion-induced sensing errors.

This event-driven approach prevents unnecessary processing when no object is present.

### 3. Material Classification Logic

Once the object is stationary:

- The inductive proximity sensor is read.

- HIGH output → Object is classified as metal.

- LOW output → Object is classified as non-metal.

This binary classification forms the first decision branch in the sorting logic.

### 4. Color Detection Logic

The system then determines the object’s color using the TCS3200 sensor:

- The sensor sequentially filters red, blue, and green light components.

- For each color filter, the output pulse frequency is measured.

- Lower pulse values correspond to higher intensity of that color.

To simplify decision-making:

- Only red and blue values are compared.

- If red intensity is higher than blue → object is classified as red.

- Otherwise → object is classified as blue.

This reduces computational complexity while maintaining reliable classification.

### 5. Decision-Making Logic (Sorting Rules)

The system combines material and color results to determine the output bin.

| Material | Color | Sorting Action |
| --- | --- | --- |
| Metal | Red | Activate Servo 1 |
| Metal | Blue | Activate Servo 2 |
| Non-Metal | Red | Activate Servo 3 |
| Non-Metal | Blue | Activate Servo 4 |

This rule-based logic ensures deterministic behavior with no overlap or ambiguity.

### 6. Actuation Logic

After classification:

- The corresponding servo motor is rotated to a fixed angle.

- This motion diverts the object into the correct bin.

- A timed delay ensures the object fully clears the sorting gate.

After actuation:

- All servos are returned to their home positions.

- The conveyor motor is restarted.

### 7. Continuous Operation Loop

The system then returns to the object detection stage and repeats the process for the next object. This loop enables:

- Continuous sorting

- Consistent timing

- Predictable mechanical behavior

### 8. Safety and Reliability Considerations

- Conveyor motion is disabled during sensing to prevent false readings.

- Servos are always reset to avoid mechanical interference.

- Modular functions isolate sensing, logic, and actuation for easier debugging.

### Summary

The system follows a sense → decide → act paradigm, which is fundamental in robotics. By combining sensor fusion with deterministic logic, the project demonstrates core robotics concepts such as real-time control, embedded decision-making, and automated actuation.